Power Calculation

V=5 voltage

Wifi active = 150mA

WiFi + Sampling Time = 200mA

Deep sleep = 12mA (* Led glow in dev Module )

On datasheet :- 8 µA

| Mode | Current |
| --- | --- |
| Deep sleep | ~8 µA |
| Active WiFi | ~150 mA |

### Assume worst case:

Active = 2 minutes per day
Sleep = rest of day

Sleep Consumption per day

8 µA × 24h ≈ 0.2 mAh

Active consumption

150 mA × (2/60 h) ≈ 5 mAh

Total per day:

≈ 5.2 mAh

### For 60 days:

5.2 × 60 = 312 mAh

# Voltage Regulation (Important)

Your ESP32 needs 3.3 V.

Non-rechargeable cells are usually:

- 4.5 V (3 × AA)

- 6.0 V (2 × CR123A)

- 7.2 V (4 × AA carbon zinc — avoid)

So you must use a voltage regulator.

| Component | What to buy |
| --- | --- |
| Battery cells | AA Lithium (2600–3000 mAh) |
| Battery holder | 4×AA holder with wires |
| Regulator | MCP1700-33 or HT7333 |
| Optional | On/Off switch |